NHTSA DEADLINE ALERT ! (In 2 Parts)

Updated JAN. 1,  2014

PART 1

MONTHLY REPORT OF EFFECTIVE DATES OF

SIGNIFICANT NEW NHTSA FINAL RULES  -- LIGHT DUTY VEHICLES

FINAL RULE& FED. REG.

DATE	(mm/dd/yy)			ISSUE				EFFECTIVE DATE

FMVSS 214 (9/11/07;			Side Impact			Phase-in

6/9/08; 3/15/10; 8/24/11)		MDB AND POLE		9/1//10-8/31/11– 20%

9/1/11-8/31/12-40%

9/1/12-8/31/13-60%

9/1/13-8/31/14-80% 9/1/14 -100%

SVMs

& LIMITED LINE (3 or fewer carlines sold in US) excluded from phase-in;

CONVERTIBLES 9/1/15 for MDB and pole test

FSM & ALTERERS – 9/1/16

VEHICLES >8500 GVWR-- 9/1/15

FMVSS 216 (5/12/09; 4/7/10)	Roof crush			Vehicles 6000lbs  							GVWR or less

Phase-in:

9/1//12-8/31/13– 25%

9/1/13-8/31/14- 50%

9/1/14-8/31/15- 75%

9/1/15 -100%

SVMs

excluded from phase-in (comply 9/1/2015)

FSM & ALTERERS – 9/1/16

Vehicles >6000 and <10K GVWR--

9/1/16 -100%

FSM & ALTERERS – 9/1/17

NHTSA FINAL RULE

& FED. REG. DATE		ISSUE				EFFECTIVE DATE

FMVSS 226 (1/19/11;2/25/11

9/9/13)					EJECTION MITIGATION

Phase-in as follows:

  - Sept 1, 2013- Aug 31, 2014 – 25%

  - Sept 1, 2014- Aug 31, 2015 –50%

  - Sept 1, 2015- Aug 31, 2016 – 75%

- Sept. 1 2016 – Aug 31, 2017 – 100% but can use advanced credits to meet phase-in %

- Sept. 1 2017 100%

- Altered vehicles and vehicles produced in more than one stage --9/1/18

- Limited-line manufacturers (3 three or fewer carlines in US) and Small manufacturers (OEM w fewer than 5,000 vehicles/yr in US) not subject to phase-in but must meet standard for prod.on or after September 1, 2017.

Distracted Driving Voluntary Guidelines – final version

April  26  2012

PART 2

MONTHLY REPORT OF

SIGNIFICANT NHTSA

PROPOSALS & UPCOMING PROPOSALS

REGARDING LIGHT DUTY VEHICLES

| NHTSA PROPOSED RULES | NHTSA EXPECTED TO BE PROPOSED RULES | NHTSA LONGER-TERM ISSUES AN OTHER PRIORITIES |
| --- | --- | --- |
| REARWARD VISIBILITY. 2008 law requires NHTSA to initiate rulemaking re expanded the required field of view to enhance rearward visibility; proposed Nov 2010; DELAYED AND NOW IN LITIGATION | Hydrogen GTR | Collision avoidance -stopped vehicle warning -active control -roadway departure warning -sonar, radar, cameras; |
| Labeling of vehicles re capable of operating on alternative fuels; |  | Tire aging |
| Decision re IIHS petition to extend bumper requirements to SUVs and pickups and vans | FMVSS 110 NPRM regarding rapid tire deflation test | EV GTR |
|  | GTR pedestrian protection | Vehicle compatibility |
| FMVSS 114 Keyless Ignition Systems Dec 2011 NPRM to require that passenger vehicles with keyless ignitions systems have consistent means to allow for a driver to stop or slow a vehicle during an emergency. | Part 543 – exemption from parts marking if certain immobilizers installed | Alternative fuel safety |
|  | FMVSS 202 TO ALIGN WITH GTR | Return of seat belt interlock? |
| Upgrade Accelerator Control Standard (FMVSS 124) - Vehicle Stopping Distance and Brake Override 4/12 NPRM to require all vehicles be equipped with a technology that would allow a Vehicle to come to a full stop with normal braking pressure when the throttle is open. Require that redundancies be built into electronic throttle control systems to enable a Driver to maintain control even if there is a failure in the system. | FMVSS 225 usability |  |
| MANDATORY EDRs NPRM published December 13 2012 |  |  |
| Designated seating position – response to petitions for reconsideration – 2013 |  |  |
| Quiet cars – NPRM published Jan 2013 |  |  |
| FMVSS 205 GTR |  |  |